CIM WIRING PINOUT

NI- 9223:		FPGA	cMODULE

VMON SIGNAL 	A0	BNC 0

ACCT SIGNAL  	A1	BNC 1

NI-9401:

SOURCE-GATE 	 	DIO0	PIN 0, COM

CHOPPER-GATE  	DIO1	PIN 1, COM

*REF to COM pins. REFER TO NI-SPEC SHEET

NI- 9422:

CHOPPER_ON 	DIO0	PIN 0+, PIN 1-

FARADAY_IN 	DIO1	PIN 2+, PIN 3-

*NEEDS + AND – WIRING. REFER TO NI-SPEC SHEET

NI-9474:

FBI			DIO0	PIN 0

*NEEDS Vsup AND GND WIRING. REFER TO NI-SPEC SHEET