Chopper Interlock Module

Specifications Design Document

Implementation in LabVIEW FPGA

| Revision: | 1.0 |
| --- | --- |
| Status: | Draft |
| Repository: | N/A |
| Project: | UPM-GMV-LIPAc Support |
| Last modification: | 29/01/2018 |
| Created: | 18/10/2017 |
| Release Date: | 29/01/2018 |

| Prepared by | Reviewed by | Approved by |
| --- | --- | --- |
| Miguel Astrain | Eduardo Barrera | Mariano Ruiz |

Table of contents

Introduction	3

CIM Variables:	3

State machine	6

CONFIGURATION:	6

PRE-ARM	6

ARMED	6

GATE OFF	7

GATE ON: RISING	7

GATE ON	7

GATE OFF: DECREASING	7

Chopper interlock Interfaces	9

USE CASES	9

CONFIGURATION	10

PRE-ARM	10

OPERATION:ARMED	10

GATE ON/OFF	11

FBI: VMON late	11

FBI: SPURIOUS ACCT	11

FBI: DUTY CYCLE	12

Requirements	12

# Introduction

The CIM module makes sure the following tasks are correctly performed and triggers and interlock action (FBI=”0”) if conditions are not met.

When the system is described as INHIBITED it means no action is taken based on plant signals, maintaining FBI=”1”. To transition into operation the CIM must be manually armed through an EPICS PV “INHIBIT”.

INHIBIT.VAL = 1 to INHIBIT operation.

INHIBIT.VAL = 0 to transition into operation.

**Caution**

All the signal names hereafter refer to the physical signals read by the CIM as depicted on the Chopper interlock Interfaces section.

| TASK | DESCRIPTION |
| --- | --- |
| T1 | Enable CIM operation only when CHOPPER_ON = “0” and FARADAY_IN =”0”. *FARADAY_IN=”1” during operation shall transition into an INHIBIT state. |
| T2 | Monitor the duty cycle and pulse length of SOURCE GATE signal with preset profiles though EPICS PV (D+ mode PV = SRC:SVL:D2OpenStat (5% duty and 3ms pulse), H+ mode PV = SRC:SVL:H2OpenStat (10% duty and 6ms pulse)). Monitor is active with CHOPPER_ON=”0”. |
| T3 | When CHOPPER_GATE is INACTIVE: - VMon should be BELOW a set threshold, VMONThr. - ACCT should be BELOW a set threshold, ACCTThr. When CHOPPER_GATE is ACTIVE: - VMon should be ABOVE a set threshold, VMONOff. - ACCT should be ABOVE the previous threshold, ACCTThr. To check this conditions some RISE and FALL times are measured, see T4 and T5. |
| T4 | Check VMON signal correctly transitions to zero within <GATETRise>microseconds of CHOPPER_GATE activation. |
| T5 | Check VMON signal correctly transitions to negative voltages within <GATETFall>microseconds after CHOPPER_GATE de-activation. |

## CIM Variables:

Configuration and output variables.

| Name | Description | LV type | Range | PV |
| --- | --- | --- | --- | --- |
| SRCDuty | Duty cycle threshold. | UInt32 | <5%,10% | --See SRCMode |
| SRCPulse | Source pulse length. | UInt32 | <3ms,6ms | -- See SRCMode |
| ACCTThr | ACCT max deviation from the bottom limit. | FXP(24,5) signed | -[16,16) | LEBT:CHOP_IM:AcctThr *PV DATATYPE: DOUBLE |
| VMONThr | VMON max deviation from the bottom limit. | FXP(24,5) signed | -[16,16) | LEBT:CHOP_IM:VmonThr *PV DATATYPE: DOUBLE |
| VMONOff | Threshold where VMON is considered “0”. | FXP(24,5) signed | -[16,16) | LEBT:CHOP_IM:VmonOff *PV DATATYPE: DOUBLE |
| VmonCGoff | Vmon value when the C.Gate signal is OFF “0”. | FXP(24,5) signed | -[16,16) | LEBT:CHOP_IM:VmonCGoff |
| StmState | State machine current state. | Enum string | STM states | LEBT:CHOP_IM:StmState |
| GATETRise | Timing window for VMON to rise after CHOPPER_GATE EDGE. | UInt8 | <15us | Not avaiblable through epicsCA. |
| GATETFall | Timing window for VMON to fall after CHOPPER_GATE EDGE. | UInt8 | <15us | Not avaiblable through epicsCA. |
| INHIBIT | Variable to manually arm the CIM. IF INHIBIT = “1” FBI=1. | Boolean | 0,1 | LEBT:CHOP_IM:Inhibit |
| FBI | Interlock output variable. | Boolean | 0,1 | LEBT:CHOP_IM:Fbi |
| SRCMode | Mode reads SRC:SVL PVs and changes CIM Duty Cycle acceptance values. | ENUM | modes | PV = SRC:SVL:D2OpenStat PV = SRC:SVL:H2OpenStat |
| Sate machine iteration | Running check. | UInt32 | STM states | LEBT:CHOP_IM:StmRunning |
| Pulse Diag | Data set which triggered FBI. | Boolean | 0,1 | LEBT:CHOP_IM:FbiDPulse |
| Duty Diag | Data set which triggered FBI. | Boolean | 0,1 | LEBT:CHOP_IM:FbiDPeriod |
| C.GATE Diag | Data set which triggered FBI. | Boolean | 0,1 | LEBT:CHOP_IM:FbiCGate |
| Faraday Diag | Data set which triggered FBI. | Boolean | 0,1 | LEBT:CHOP_IM:FbiFaraday |
| ACCT Diag | Data set which triggered FBI. | FXP(24,5) signed | -[16,16) | LEBT:CHOP_IM:FbiAcct |
| VMon Diag | Data set which triggered FBI. | FXP(24,5) signed | -[16,16) | LEBT:CHOP_IM:FbiVmon |
| State Diag | Data set which triggered FBI. | String | STM states | LEBT:CHOP_IM:FbiStmState |

Reuqested on-site, the CIM also publishes the following PV’s while in the PRE-ARM state for debugging purposes:

| Pulse Debug | PRE-ARM values. | UInt32 | System ticks(25ns) | LEBT:CHOP_IM:DbgDPulse |
| --- | --- | --- | --- | --- |
| Duty Debug | PRE-ARM values. | UInt32 | System ticks(25ns) | LEBT:CHOP_IM:DbgDPeriod |
| ChopperON Debug | PRE-ARM values. | Boolean | 0,1 | LEBT:CHOP_IM:DbgChop |
| Faraday Debug | PRE-ARM values. | Boolean | 0,1 | LEBT:CHOP_IM:DbgFaraday |
| ACCT Debug | PRE-ARM values. | FXP(24,5) signed | -[16,16) | LEBT:CHOP_IM:DbgAcct |
| VMon Debug | PRE-ARM values. | FXP(24,5) signed | -[16,16) | LEBT:CHOP_IM:DbgVmon |

# State machine

To manage the tasks the following state-machine is proposed:

## CONFIGURATION:

- In this state correct configuration is checked:

- Module configurations

- Threshold values

- FBI=1

- The system transitions into the next state when the following conditions are met:

- No Errors.

## PRE-ARM

- The CIM stays in this state until armed for operation. It will maintain FBI state as it was when entering this state.

- If the system enters PRE-ARM from CONFIGURATION, the FBI =” 1” = ALL OK.

- If the system enters PRE-ARM from an interlock action FBI=”0” it stays there until re-armed.

- The system transitions into the next state when the following conditions are met:

- Arming the chopper is done manually though an EPICS PV: CIM:INHIBIT = 0.

- CIM defaults to INHIBIT=”1” on start up. Needs to be manually armed through EPICS PV.

## ARMED

FBI = “1” is set regardless of previous value.

The CIM then waits for CHOPPER_ON = “0” and FARADAY_IN = “0” to transition into the GATE OFF state.

## GATE OFF

CIM monitors the following signals in this state:

- SOURCE_GATE: Duty cycle <SRCDuty; Pulse <SRCPulse.

- ACCT: Voltage <ACCTThr

- VMON: Voltage <VMONThr

- CHOPPER_ON

- FARADAY_IN

- CHOPPER_GATE

CIM waits in this state until a rising edge is detected on the CHOPPER_GATE signal.

**Modified on-site operation**

Faraday cup insertion during a GATE_OFF cycle shall force a transition into the PRE-ARM state, in more detail:

- FARADAY_IN=”1” (with the remaining signals on the correct values, meaning:

- CHOPPER_ON=”0” (the physical signal received by the CIM)

- CHOPPER_GATE=”0

- and thresholds as indicated above

this combination shall INHIBIT CIM and transition into a PRE-ARM state. This was requested by the user to be able to “disarm” the CIM without triggering an interlock action.

## GATE ON: RISING

CIM stays for (GATETRise) microseconds on this state.

CIM monitors the following signals in this state:

- SOURCE_GATE: Duty cycle <SRCDuty; Pulse <SRCPulse.

- CHOPPER_GATE

- VMON: Voltage

- CHOPPER_ON

- FARADAY_IN

CIM monitors VMON voltage until GATETRise time has passed, if VMON is below (in absolute value) the VMONOff value it shall transition to the next state.

## GATE ON

CIM monitors the following signals in this state:

- SOURCE_GATE: Duty cycle <SRCDuty; Pulse <SRCPulse.

- CHOPPER_GATE

- VMON: Voltage

- CHOPPER_ON

- FARADAY_IN

In this state CIM monitors VMON is below (in absolute value) the VMONOff value and waits for a falling edge on CHOPPER_GATE when it transitions onto the next state.

## GATE OFF: DECREASING

CIM stays for (GATETFall) microseconds on this state.

CIM monitors the following signals in this state:

- SOURCE_GATE: Duty Cycle <SRCDuty; Pulse <SRCPulse.

- CHOPPER_GATE

- VMON: Voltage

- CHOPPER_ON

- FARADAY_IN

CIM monitors VMON voltage until GATETFall time has passed, if VMON is over (in absolute value) the VMONThr value it shall transition back to GATE_OFF.

## Chopper interlock Interfaces

The interfaces are realized as follows:

| SIGNAL | Type | Range |
| --- | --- | --- |
| CHOPPER_ON | DI 24V | 0-24 V |
| FARADAY_IN | DI 24V | 0-24 V |
| SOURCE_GATE | DI TTL | 5V TTL |
| CHOPPER_GATE | DITTL | 5V TTL |
| VMON | ANALOG | [-4,0] V |
| ACCT | ANALOG | [-2,2] V *ACCT sensor still uncertain. |
| FBI | DO 24V | 0-24 V |

# USE CASES

The following use cases illustrate some of the Interlock behavior:

## CONFIGURATION

## PRE-ARM

## OPERATION:ARMED

## GATE ON/OFF

## FBI: VMON late

## FBI: SPURIOUS ACCT

## FBI: DUTY CYCLE

# Requirements

CIM-GNRL-000

Chopper Interlock Module (CIM hereafter) shall be implemented using NI-9068 RT chassis. Interfaces shall be implemented using NI-cRIO series Modules and in compliance with their electrical compatibility specifications.

CIM-GNRL-001

CIM shall be developed using:

- LabVIEW 2014 SP1 f10

  - LabVIEW-RT 2014

  - LabVIEW DSC 2014

  - LabVIEW FPGA 2014

- LabVIEW Xilinx vivado tools 13

- NI-RIO 14.0.1

CIM-GNRL-002

CIM shall publish Process Variables (PV) to EPICS using LabVIEW-RT EPICS IOC and Shared Variable PV types.

CIM-FUNC-000

CIM shall be implemented using State Machine design scheme.

CIM-FUNC-001

CIM operation shall be enabled by the actuation of two digital 24V signals. CHOPPER_ON and FARADAY_IN.

CIM-FUNC-002

When enabled, CIM shall monitor a periodic TTL signal SOURCE_GATE.

CIM-FUNC-003

When enabled, CIM shall monitor two analog signals ACCT and VMON.

CIM-FUNC-004

When CHOPPER_GATE is enabled, CIM shall check the correct transition of the VMON signal.

CIM-FUNC-005

When CHOPPER_GATE is disabled, CIM shall check the correct transition of the VMON signal.

CIM-FUNC-006

CIM shall trigger an "interlock action" when some conditions are met.

The "interlock action" is realized with a digital 24V signal on a logic zero and latching onto this state.

CIM shall need to be re-armed from to continue operation.

CIM-TEST-000

CIM Factory Acceptance Tests shall be carried out using real signals generated using NI Multifunction DAQ 6361.

CIM-TEST-001

CIM Site Acceptance tests shall follow the requirements specifications.